APS

Advanced Planning & Scheduling

Steel / Metal Plant

System Guide

April 2026

# 1. What Is This?

This is a Finite Scheduling System for an integrated steel plant with Blast Furnace, Steel Melt Shop (2 EAF + 3 LRF + 1 VD + 2 CCM), and 2 Rolling Mills. It takes your sales orders, groups them into campaigns, figures out what raw materials and capacity you need, checks material availability, and generates a constrained production schedule that tells each machine exactly what to produce and when.

The system lives in an Excel workbook connected to a Python engine. You interact with Excel (view data, click buttons). Python does the heavy computation (campaign building, BOM explosion, capacity analysis, finite scheduling) and writes results back into Excel.

Current runtime stack: pandas for data shaping, xlwings + pywin32 for Excel connectivity, openpyxl for workbook generation, and OR-Tools CP-SAT for finite scheduling. The plant configuration models 11 resources with a 50 MT EAF heat size.

How It Works:

| Sales Orders | BOM Explosion | Capacity Map | Scheduler | Schedule Output |
| --- | --- | --- | --- | --- |
| What do customers want? | What materials do we need? | Can our machines handle it? | What runs where & when? | Final production plan |

# 2. File Structure

All files live in a single folder. Here is what each file does:

| File / Folder | Purpose |
| --- | --- |
| APS_BF_SMS_RM.xlsm | The Excel workbook. All data input, buttons, and output sheets. |
| archive/legacy_single_workbook_path/ | Retired APS_Steel_Template workbook flow and old root-level scripts. Kept only for reference. |
| aps_functions.py | Bridge between Excel buttons and Python engine (xlwings). |
| run_all.py | Run full pipeline from command line: python run_all.py |
| engine/bom_explosion.py | Explodes multi-level BOM, nets against inventory. |
| engine/capacity.py | Maps demand to machine hours, compares vs available capacity. |
| engine/campaign.py | Campaign builder: groups SOs into release buckets, consumes FG stock, checks material availability via BOM. |
| engine/scheduler.py | Finite scheduler using OR-Tools CP-SAT, with greedy fallback only if the solver cannot produce a usable schedule. |
| scenarios/scenario_runner.py | Runs 4 base what-if scenarios and adds extra stress cases when yield loss, rush order, or overtime controls are populated. |
| data/loader.py | Reads Excel file into pandas DataFrames for standalone testing. |
| setup_excel.py | One-time setup: embeds VBA + creates buttons in workbook. |
| build_template.py | One-time: generates the canonical APS_BF_SMS_RM workbook with dummy steel data. |

Only one workbook flow is supported: APS_BF_SMS_RM.xlsm. Older APS_Steel_Template files and their root-level scripts are archived under archive/legacy_single_workbook_path and are not part of the live runtime.

# 3. Excel Sheets Explained

## Input Sheets (you fill these)

| Sheet | What It Contains | Key Columns |
| --- | --- | --- |
| Help | Dedicated workbook reference tab. Inline sheet guides are removed from the working tabs. | Sheet, Type, What It Is For, How To Use It |
| SKU_Master | Finished goods, RM outputs, billets, EAF/LRF/VD/BF intermediates, raw materials, and byproduct/waste SKUs | SKU_ID, Category, Grade, Section_mm, Needs_VD |
| BOM | Stagewise Bill of Materials with input and byproduct rows from FG down to BF/raw materials | Parent_SKU, Child_SKU, Flow_Type, Qty_Per, Scrap_% |
| Inventory | Current stock by SKU and location, including billets, hot metal, WIP stages, and optional waste sinks | SKU_ID, Available_Qty, Reserved_Qty |
| Sales_Orders | Customer orders with quantities and delivery dates | SO_ID, SKU_ID, Order_Qty, Delivery_Date |
| Resource_Master | Machines: EAF, CCM, Rolling Mill, etc. | Resource_ID, Avail_Hours_Day, Efficiency_% |
| Routing | How each SKU flows through machines (operation sequence) | SKU_ID, Operation_Seq, Resource_ID, Cycle_Time |
| Changeover_Matrix | Time (minutes) to switch between products on a machine | From SKU x To SKU matrix |
| Scenarios | Active planning controls plus what-if parameters | Demand Spike, Machine Down, Start Hour, Solver Limit, Yield Loss |

The workbook now uses one dedicated Help sheet instead of right-side inline Sheet Guide panels on every working tab. This keeps the planning sheets visually cleaner.

## Output Sheets (Python fills these)

| Sheet | What Gets Written |
| --- | --- |
| BOM_Output | BOM explosion results: gross requirements, available inventory, and net requirements per material |
| Capacity_Map | Demand hours vs available hours per machine. Shows utilisation %, flags overloaded/underutilised. |
| Schedule_Output | Detailed dispatch schedule: one row per planned operation on one resource with start/end time, heat no, quantity, and status. |
| Campaign_Schedule | Campaign-level summary across EAF, CCM, and RM stages for planner and management review. |
| Material_Plan | Campaign-by-campaign material commitment trace showing what inventory was consumed, what remains, and what caused any hold. |
| Equipment_Schedule | Separate dispatch tables per equipment with blank rows between sections for cleaner operational handoff. |
| Schedule_Gantt | Resource swim-lane Gantt with adaptive 1/2/4-hour campaign bars, plant-separated lanes, and utilisation summary per machine. |
| Scenario_Output | Base scenario comparison plus optional stress cases, showing service, lateness, throughput, bottleneck, and utilisation KPIs. |
| KPI_Dashboard | Chart-based APS dashboard: KPI tiles plus 6 charts for utilisation, campaign outcome, operation mix, throughput, and scenario comparison. |
| Theo_vs_Actual | Placeholder: compare planned vs actual process times once production data flows in. |

# 4. How Scheduling Works

## Step-by-Step Pipeline

Step 1: Read Inputs and Scenario Controls

The APS engine reads sales orders, inventory, BOM, resources, routing, and scenario-control values from Excel. Demand spike, downtime hours and start offset, planning horizon, solver time limit, yield loss, rush order, and overtime are applied before the scheduling model is built.

Step 2: Normalize Open Demand and Consume FG Stock

Only open sales orders are considered. Finished-good inventory is consumed first so only uncovered make quantity becomes production demand. This keeps the schedule focused on what the plant still needs to produce.

Step 3: Build Compatible Campaigns

The current campaign engine groups compatible orders into one PPC release batch based on Campaign Group, Grade, Billet Family, and VD requirement. The batch is then split by campaign-size rules. Each campaign contains linked RM production orders derived from the covered sales orders.

Step 4: Material Release Check

Each campaign is checked against BOM and inventory before it is released to scheduling. If raw materials or intermediates are short, the campaign is marked MATERIAL HOLD and does not enter the finite schedule.

Step 5: Build the Finite Scheduling Model

For released campaigns, the engine builds SMS heat tasks and RM production-order tasks. Routing times are used where available, and running rows that planners marked as RUNNING are frozen so their timing and resource assignment stay pinned on rerun.

Step 6: Apply Process Precedence

Within each campaign, SMS precedence is modeled as EAF -> LRF -> VD? -> CCM. RM production orders are then allowed to start only after the campaign's CCM work is complete, and the solver can choose among alternate machines in each resource group.

Step 7: Enforce Campaign Serialization

Campaigns are treated as PPC release batches, not mix-and-match machine buckets. Campaign n+1 cannot start until campaign n is fully complete end-to-end. That means the plant should not switch to the next campaign on CCM, RM, or any other modeled unit before the current campaign is finished.

Step 8: Apply Machine and Downtime Constraints

The model enforces no-overlap on each machine calendar, inserts a blocking interval for the selected machine-down resource at the configured downtime start hour, and applies partial RM changeover gaps from the Changeover_Matrix. Steel flow is planned back-to-back without an inserted artificial safety buffer.

Step 9: Solve and Fallback

The OR-Tools CP-SAT model is solved with a configurable time limit and lateness pressure on both SMS completion and RM completion. If the exact model becomes infeasible, the engine falls back to the greedy scheduler so the workbook still receives a usable plan.

Step 10: Write Outputs and Dashboards

The run writes machine-level rows to Schedule_Output, campaign rows to Campaign_Schedule, material commitments to Material_Plan, grouped packets to Equipment_Schedule, a resource swim-lane view to Schedule_Gantt, and charts plus KPI tiles to KPI_Dashboard.

## Campaign Building Logic

The campaign engine (engine/campaign.py) is central to the APS. Campaigns are the unit of PPC release and execution — every scheduling decision flows through them:

- Open SOs are prioritized by Priority (URGENT > HIGH > NORMAL > LOW), then Delivery_Date, then Order_Date.

- Finished-good inventory is consumed first so only uncovered make quantity becomes production demand.

- Compatible SOs are grouped by Route Family = Campaign_Group + Grade + Billet_Family + VD requirement.

- Groups exceeding Max Campaign MT (default 500) are split. Below-minimum campaigns are flagged.

- Each campaign generates derived production orders for the RM. Heats are calculated as ceil(total_liquid_MT / 50).

- Released campaigns are serialized. The next campaign cannot begin until the previous campaign is fully complete end-to-end.

- Material availability is checked via BOM explosion per campaign. Shortages result in MATERIAL HOLD status.

## Important Note on Constraint Evaluation

The planning pipeline above happens in a sequence. The solver itself does not evaluate constraints one-by-one in that same sequence. Once the CP-SAT model is built, active constraints are solved together as one optimization problem.

So the row order in Schedule_Output is a display order for planners, not the order in which constraints were checked by the solver.

# 5. How to Use the System

## Option A: Click Buttons in Excel

Open APS_BF_SMS_RM.xlsm. You can launch actions from the Control_Panel or from the relevant working sheets such as BOM, Scenarios, Capacity_Map, Schedule_Output, Campaign_Schedule, Material_Plan, Equipment_Schedule, Schedule_Gantt, BOM_Output, and Scenario_Output.

| Button | What It Does |
| --- | --- |
| Run BOM Explosion | Explodes demand through BOM, nets against inventory, writes to BOM_Output |
| Run Capacity Map | Calculates machine hours needed vs available, writes to Capacity_Map sheet |
| Run Schedule | Generates the live production schedule, writes to Schedule_Output, Campaign_Schedule, Material_Plan, Equipment_Schedule, and Schedule_Gantt |
| Run Scenarios | Runs 4 base scenarios plus optional extra stress cases, writes comparison to Scenario_Output |
| Clear All Outputs | Clears all computed results from output sheets |

## Option B: Run from Command Line

With the Excel file open, run from a terminal:

python run_all.py

python run_all.py bom

python run_all.py capacity

python run_all.py schedule

python run_all.py scenarios

This connects to the open Excel workbook via xlwings and writes results directly into the sheets.

Do not run the buttons from any archived APS_Steel_Template workbook. Those files are retained only for historical reference.

# 6. What-If Scenarios

The system always runs 4 base scenarios to stress-test your plan, and it can add extra stress cases when yield loss, rush order, or overtime controls are populated:

The editable values on the Scenarios sheet also affect the live APS plan when you rerun BOM, Capacity, or Schedule.

| Scenario | What Changes |
| --- | --- |
| Baseline | Current demand and full machine availability. Your reference point. |
| Demand +15% | All order quantities increased by 15%. Shows which machines break first. |
| EAF-01 Down 8hrs | Electric Arc Furnace 1 loses 8 hours. Tests bottleneck impact. |
| Demand +15% + EAF Down | Both stress factors combined. Worst-case planning. |
| Yield Loss | Appears when Yield Loss (%) is non-zero. Increases required liquid steel and heats. |
| Rush Order | Appears when Rush Order MT is non-zero. Injects urgent demand into the planning pool. |
| Extra Shift | Appears when Extra Shift Hours is non-zero. Adds overtime hours to all resources. |

You can modify scenario parameters in the Scenarios sheet before running. Important controls include Demand Spike, Machine Down Hours and Start, Solver Time Limit, Yield Loss, Rush Order MT, Extra Shift Hours, and campaign-size limits. If you manually set a row in Schedule_Output to RUNNING, reruns preserve that row instead of rewriting it.

# 7. Dummy Data (Steel Context)

The template comes pre-loaded with realistic steel plant data:

- 21 Finished Goods: Wire Rod Coils across 8 grades (SAE 1008/1018/1035/1045/1065/1080, CHQ 1006, Cr-Mo 4140) and 5 sections (5.5/6.5/8.0/10.0/12.0 mm)

- Stage intermediates: RM outputs, CCM billets, EAF liquid steel, LRF output, VD output for selected grades, BF hot metal, and BF raw-mix burden

- Tracked byproducts and waste: BF slag, EAF slag, LRF waste, VD waste, CCM crop, RM end cuts, and RM scale

- 11 Machines: 1 BF, 2 EAFs, 3 LRFs, 1 VD, 2 CCMs, 2 Rolling Mills — EAF heat = 50 MT

- 30 Sales Orders from Indian wire/steel companies (Tata Wiron, Usha Martin, JSW Steel, etc.)

- Stagewise BOM: FG -> RM Output -> CCM Billet -> LRF/VD Output -> EAF Output -> EAF Charge -> BF Hot Metal / Raw Materials, with byproducts tagged separately

Replace this data with your actual plant data when ready. The structure and column names must remain the same.

# 8. Prerequisites & Setup

## Software Required

| Software | Version | Purpose |
| --- | --- | --- |
| Python | 3.11+ (Windows) | Runs the scheduling engine |
| Excel | 2016+ or 365 | User interface and data entry |
| xlwings | pip install xlwings | Excel-Python bridge |
| pywin32 | pip install pywin32 | Windows COM bridge used by setup and button wiring |
| pandas | pip install pandas | Data manipulation |
| openpyxl | pip install openpyxl | Excel file generation |
| ortools | pip install ortools | Finite scheduling optimizer |

## One-Time Setup Steps

- Install Python packages: pip install xlwings pywin32 pandas openpyxl ortools

- Install xlwings Excel add-in: xlwings addin install

- In Excel: File > Options > Trust Center > Trust Center Settings > Macro Settings > enable "Trust access to the VBA project object model"

- Run: python setup_excel.py (embeds VBA and creates buttons)

- In xlwings ribbon: set Interpreter to your python.exe path, set PYTHONPATH to the APS folder

- Click any button on Control_Panel to verify everything works

# 9. Technical Gaps & Next Steps

| # | Gap | Why It Matters / Next Upgrade |
| --- | --- | --- |
| 1 | Machine-choice objective quality | Alternate-machine assignment is active, but the objective still needs stronger load balancing, anti-fragmentation, and symmetry-breaking across parallel units. |
| 2 | Changeover optimization | RM same-machine changeovers are active, but full steel sequencing, caster-aware changeovers, and richer metallurgical sequence quality are still simplified. |
| 3 | True steel process continuity | CCM continuity, tundish changes, strand-level sequencing, and upstream BF constraints are still simplified. |
| 4 | Planner-facing campaign governance | Campaigns are auto-built compatible release buckets today. A stronger APS should support planner approval, campaign boards, and explicit per-SO allocated tonnage. |
| 5 | Calendars and maintenance | The model supports available hours and a simple downtime event, but not full shift calendars, crew calendars, or maintenance calendars. |
| 6 | ERP/MES integration | Production orders and execution feedback still live mainly in Excel. A stronger APS needs an external system of record and interface layer. |
| 7 | Actual-vs-plan feedback | Theo_vs_Actual is ready as a placeholder, but the system still needs actual shop-floor feedback loops for closed-loop planning. |
| 8 | Validation and observability | The project still needs stronger tests, schema validation, and run logging to be fully production-grade. |

# 10. Scheduling Constraints (Current & Planned)

| Constraint | Status | How It Works |
| --- | --- | --- |
| Routing-based durations | Active | Uses Routing timings where available, with defaults as fallback |
| Due-date pressure | Active | Lateness is tracked and the solver tries to keep campaigns on time |
| No machine overlap | Active | One task per modeled machine at a time |
| Alternate-machine assignment | Active | Optional intervals let the solver choose one machine from each resource pool |
| Artificial buffer between jobs | Removed | Steel flow is planned back-to-back without an inserted safety gap |
| Late job detection | Active | Campaigns finishing after due date are marked LATE |
| Operation precedence | Active | EAF before LRF before VD before CCM; RM starts only after CCM completion |
| Machine down interval | Active | Selected downtime resource is blocked for the configured outage duration starting at the chosen offset |
| Running-order freeze | Active | Rows marked RUNNING are pinned and not rescheduled on rerun |
| Material release gate | Active | Campaigns with shortages are held before they enter finite scheduling |
| Changeover duration | Partial | RM same-machine gaps use the Changeover_Matrix, but full steel sequencing is still simplified |
| Customer grouping | Planned | Ship all orders for same customer together |
| Region optimization | Planned | Group by delivery location/region |
| Machine-choice load balancing | Planned | Needed to balance work more intentionally across parallel units once assignment is already active |